MATERIAL SAFETY DATA SHEET

Abbreviations used on this Material Safety Data Sheet:  
N/av. = Not available, N/ap. = Not applicable, ppm = parts per million, TLV = Threshold Limit Value.

NFPA Hazard Rating: 4 - Extreme, 3 - High, 2 - Moderate, 1- Slight, 0- None, X - Blank

WHMIS HAZARD: SLIGHT  NFPA HAZARD RATING:  NONE  REACTIVITY –0  HEALTH - 1 , FLAMMABILITY - 0 ,

| SECTION 1 – PRODUCT IDENTIFICATION AND USE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PRODUCT IDENTIFIER: |  |  |  |  |  |  |  |  | DECA BRITE (Liquid) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PRODUCT USE: |  |  |  |  |  |  |  |  | CARPET AND UPHOLSTERY CLEANER |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MANUFACTURER’S NAME: |  |  |  |  |  |  |  |  | CHEM-TECH INDUSTRIES LTD. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SUPPLIERS NAME: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STREET ADDRESS: |  |  |  |  |  |  |  |  | 3640 WESTON ROAD UNIT 12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | STREET ADDRESS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CITY: |  |  | TORONTO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PROVINCE/STATE: |  |  |  |  |  |  |  | ON |  |  |  | CITY: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PROVINCE: |  |  |  |  |  |  |
| POSTAL CODE: |  |  |  | M9L 1W2 |  |  |  |  |  |  |  | EMERGENCY TELEPHONE: |  |  |  |  |  |  |  | 1-800-535-5053 |  |  |  |  |  |  |  |  |  |  |  | POSTAL CODE: |  |  |  |  |  |  |  |  | EMERGENCY TELEPHONE: |  |  |  |  |  |  |  |  |  |  |  |  |
| SECTION 2 – HAZARDOUS INGREDIENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HAZARDOUS INGREDIENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | % |  |  |  | CAS NUMBER |  |  |  | LETHAL DOSE (50%) OF INGREDIENT (SPECIFY SPECIES AND ROUTE) |  |  |  |  |  |  |  |  |  |  | LETHAL CONCENTRATION (50%) OF INGREDIENT (SPECIFY SPECIES AND ROUTE) |  |  |  |  |  |  |  |  |  |
| DENATURED ALCOHOL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |  | 64-17-5 |  |  |  | 3450 MG/KG ORAL MOUSE |  |  |  |  |  |  |  |  |  |  | 2000 PPM/10 H |  |  |  |  |  |  |  |  |  |
| C9-C11 LINEAR PRIMARY ALCOHOL ETHOXYLATE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1-5 |  |  |  | 68439-46-3 |  |  |  | 1400 MG/KG (ORAL/RAT) |  |  |  |  |  |  |  |  |  |  | N/AV |  |  |  |  |  |  |  |  |  |
| 2-PROPANOL 1-PROPOXY / n-PROPOXYPROPANOL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6 |  |  |  | 1569-01-3 |  |  |  | 2920 mg/kg (oral/rat) |  |  |  |  |  |  |  |  |  |  | N/AV |  |  |  |  |  |  |  |  |  |
| WATER |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 |  |  |  | 7732-18 |  |  |  | N/AV |  |  |  |  |  |  |  |  |  |  | N/AV |  |  |  |  |  |  |  |  |  |
| SECTION 3 – PHYSICAL DATA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHYSICAL STATE: |  |  |  |  |  |  | LIQUID |  |  |  |  |  | ODOUR AND APPEARANCE: |  |  |  |  |  |  |  |  |  | NATURAL CITRUS ODOR; CLEAR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | THRESHOLD ODOUR (ppm): |  |  |  |  | N/AP |  |  |  |
| VAPOUR PRESSURE: (mm Hg) |  |  |  |  |  | 1.7mm Hg |  |  |  |  |  |  | VAPOUR DENSITY: (Air = 1) |  |  |  |  |  |  |  |  | 4.1 |  |  |  |  | EVAPORATION RATE:(n-Butyl acetate = 1) |  |  |  |  | 0.22 |  |  |  | BOILING POINT: |  |  |  |  |  |  | >100 ºC |  | FREEZING POINT: |  |  |  |  | <0 ºC |  |  |  |
| PH: |  | 7.5-8.5+0.5 |  |  |  |  |  |  |  |  |  |  | SPECIFIC GRAVITY: (Water = 1) |  |  |  |  |  |  |  |  | 1.02+.01 |  |  |  |  | COEFFICIENT OF WATER/OIL DISTRIBUTION: |  |  |  |  |  |  |  | 100% SOLUBLE IN WATER |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SECTION 4 – FIRE AND EXPLOSION DATA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FLAMMABILITY IF YES, UNDER WHICH CONDITIONS? YES NO X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MEANS OF EXTINCTION: |  |  |  |  |  |  |  |  |  |  | AS FOR SURROUNDING FIRE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SPECIAL PROCEDURES: |  |  |  |  |  |  |  |  |  |  | NORMAL FIRE FIGHTING PROCEDURES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FLASHPOINT AND METHOD: |  |  |  |  |  |  |  |  | N/AP |  |  |  |  |  |  |  |  |  |  |  |  |  |  | UPPER FLAMMABILITY LIMIT: (% BY VOLUME) |  |  |  |  |  |  |  | N/AP |  |  |  |  |  |  |  | LOWER FLAMMABILITY LIMIT: (% BY VOLUME) |  |  |  |  |  |  |  | N/AP |  |  |  |  |  |
| AUTOIGNITION TEMPERATURE: |  |  |  |  |  |  |  |  | NONE |  |  |  |  |  |  |  |  |  |  |  |  |  |  | HAZARDOUS COMBUSTION PRODUCTS: |  |  |  |  |  |  |  | NONE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EXPLOSION DATA: SENSITIVITY TO IMPACT: |  |  |  |  |  |  |  |  | NO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SENSITIVITY TO STATIC DISCHARGE: |  |  |  |  |  |  | NO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SECTION 5 – REACTIVITY DATA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHEMICAL STABILITY IF NO, UNDER WHICH CONDITIONS? YES X NO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| INCOMPATIBILITY WITH OTHER SUBSTANCES IF YES, WHICH ONES? YES X NO STRONG OXIDIZING AGENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| REACTIVITY, AND UNDER WHAT CONDITIONS? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NONE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HAZARDOUS DECOMPOSITION PRODUCTS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NONE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PRODUCT IDENTIFIER: |  |  |  |  |  |  | DECA BRITE (LIQUID) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SECTION 6 –TOXICOLOGICAL PROPERTIES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ROUTE OF ENTRY: |  |  |  | SKIN CONTACT |  |  |  |  | X |  |  |  |  | SKIN ABSORPTION |  |  |  |  |  |  |  |  |  |  | EYE CONTACT |  |  |  |  | X |  |  | INHALATION |  |  |  |  |  |  |  |  |  |  |  | INGESTION |  |  |  |  | X |  |  |
|  | EFFECTS OF ACUTE EXPOSURE TO PRODUCT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | INHALATION,EYE AND SKIN CONTACT MAY CAUSE IRRITATION. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | EFFECTS OF CHRONIC EXPOSURE TO PRODUCT: PROLONGED OR REPEATED EXPOSURE CAN CAUSE DRYING, DEFATTING AND DERMATITIS OF THE SKIN. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | EXPOSURE LIMITS: |  |  |  |  |  |  |  |  |  |  |  |  | IRRITANCY OF PRODUCT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SENSITIZATION TO PRODUCT: |  |  |  |  |  |  |  |  |  |  |  | CARCINOGENICITY: |  |  |  |  |  |  |  |  |  |  |  |
|  | N/AV |  |  |  |  |  |  |  |  |  |  |  |  | EYE AND SKIN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/AV |  |  |  |  |  |  |  |  |  |  |  | NO |  |  |  |  |  |  |  |  |  |  |  |
|  | TERATOGENICITY: |  |  |  |  |  |  |  |  |  |  |  |  | REPRODUCTIVE TOXICITY: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MUTAGENICITY: |  |  |  |  |  |  |  |  |  |  |  | SYNERGISTIC PRODUCTS: |  |  |  |  |  |  |  |  |  |  |  |
|  | N/AV |  |  |  |  |  |  |  |  |  |  |  |  | N/AV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/AV |  |  |  |  |  |  |  |  |  |  |  | N/AV |  |  |  |  |  |  |  |  |  |  |  |
|  | SECTION 7 – PREVENTIVE MEASURES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PERSONAL PROTECTIVE EQUIPMENT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | GLOVES (SPECIFY): |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RESPIRATOR (SPECIFY): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | EYE (SPECIFY): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NEOPRENE OR RUBBER |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FOR SPILLS OR OVEREXPOSURE NIOSH APPROVED, ORGANIC VAPOUR CARTRIDGE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CHEMICAL GLASSES OR GOGGLES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | FOOTWEAR (SPECIFY): |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CLOTHING (SPECIFY): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OTHER (SPECIFY): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IMPERVIOUS |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/AP |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ENGINEERING CONTROLS (SPECIFY, VENTILATION, ENCLOSED PROCESS): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | GENERAL VENTILATION NORMALLY ADEQUATE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | LEAK AND SPILL PROCEDURE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RINSE WITH WATER AND LET DRY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | WASTE DISPOSAL: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ACCORDING TO LOCAL, PROVINCIAL, AND FEDERAL GOVERNMENT REQUIREMENTS. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | HANDLING PROCEDURES AND EQUIPMENT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | KEEP CONTAINER TIGHTLY CLOSED WHEN NOT IN USE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | STORAGE REQUIREMENTS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | STORE IN WELL VENTILATED AREA. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SPECIAL SHIPPING INFORMATION: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TDG. UN NUMBER |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NORMAL PROCEDURES; KEEP FROM FREEZING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/AP |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SECTION 8 – FIRST AID MEASURES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SPECIFIC MEASURES: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CONTACT WITH EYES: FLUSH WITH COPIOUS AMOUNTS OF POTABLE WATER. SEEK MEDICAL ATTENTION IMMEDIATELY. CONTACT WITH SKIN: FLUSH WITH COPIOUS AMOUNTS OF POTABLE WATER. IF IRRITATION PERSISTS, SEEK MEDICAL ATTENTION IMMEDIATELY. INGESTION: DO NOT INDUCE VOMITING. GIVE LOTS OF WATER OF MILK. SEEK MEDICAL ATTENTION. INHALATION: REMOVE TO FRESH AIR. IF DISCOMFORT PERSISTS OBTAIN MEDICAL ATTENTION. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SECTION 9 – PREPARATION DATE OF MSDS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PREPARED BY: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PHONE NUMBER: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DATE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CHEM-TECH INDUSTRIES LTD. DIRECTOR TECHNICAL SERICES |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1-416-450-8324 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | APRIL 2010 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |